A-OldBoard hard coded speed of 50 mL/min

Screen at power up for older program    SyringePump200mL.ino

            - First Time pressing Select/Menu button:  This is the BolusStep Menu

- Left and right do nothing on this menu but  UP or Down cycle through BolusStep options here did the down button once.

3.Second  time push Select/Menu Button:

4. In above menu screen Left (Pull), Right (Push) from the above screen.

5.In above Menu you can change the total Bolus to  sample by using up or down buttons on this screen

6. Note Used  at the top of the screen will only update if used in push mode if you are pulling it remains 0.000

Old Key Pad Board can select 10 mL/min, 50mL/min or 400mL/min speeds

Open Screen Display:

- Syringe Pump V2.0  Rob’s original code with extraneous code, old board speed =50 mL/min

- SyringePumpV3 MR: finalized 7/14/2020 at 10:11am Maritza  Romero

Same execution as v2.0 with Hard coded speed currently at 50mL/min  (old board)

Old Board removed all sections of code (like triggers and Serial) that did nothing and commented code.   Program hard coded speed of syringe in line below.

#define SPEED_MICROSECONDS_DELAY 450 // (450 = 50mL/min for 300mL syringe)

If you want  to slow down to 10mL/min change 450 to 2250 on the above line of code

- SyringePump  (old board with selectable  syringe speed included extraneous code and errors)

- SyringePumpV4  still using old board removed extraneous code and corrected errors allows selection of syring speed onscreen

Opening screen V 4 Syringe Pump:

- First time press menu key

- Up or down to  select speed you want to use

- Once speed selected press the Menu button again

- Up and down to change the volume you want to push or sample

- From the above screen push left to pull sample or right to push a volume

New Board : https://www.adafruit.com/product/772